SECTION 08370 (08 37 00)

GARAGE DOORS

Display hidden notes to specifier. (Don't know how? Click Here)

Copyright 2024 ARCAT, Inc. - All rights reserved

** NOTE TO SPECIFIER ** C.H.I. Overhead Doors; Overhead Doors.

This section is based on the products of C.H.I. Overhead Doors, which is located at:
1485 Sunrise Drive.
Arthur, IL 61911.
Tel:  (800) 677-2650.
Fax:  (800) 738-5006.
E-mail:  aia@chiohd.com
Web:  www.chiohd.com.

[Click Here] for more information

C.H.I. Overhead Doors, a NUCOR (NYSE: NUE) company, has been manufacturing overhead doors for over 40 years. Through our authorized dealer network across North America, you can access our entire product line including commercial and residential sectional doors, rolling service and fire doors or shutters, and high-performance doors. C.H.I. integrates premium-quality materials with superior designs, workmanship, and a strong focus on end user satisfaction. Dedicated to continuing the best customer service and dealer support in the industry, it is apparent why C.H.I. is referred to as “The Door to Quality". C.H.I. is headquartered in Arthur, IL with additional manufacturing in Terre Haute, IN. For more information visit chiohd.com.

GENERAL

SECTION INCLUDES

RESIDENTIAL INSULATED STEEL PAN STYLE DOORS

RELATED SECTIONS

** NOTE TO SPECIFIER ** Delete any sections below not relevant to this project; add others as required.

Section 05 10 00 - Structural Metal Framing.

Section 06 10 00 - Rough Carpentry.

Section 09 90 00 - Painting and Coating.

Section 26 05 00 - Common Work Results for Electrical.

REFERENCES

** NOTE TO SPECIFIER ** Delete references from the list below that are not required by the text of the edited section.

America National Standards Institute (ANSI) / Door & Access Systems Manufacturers Association, International (DASMA):

DASMA Thermal Performance Verification Program.

ANSI/DASMA 105 - Test Method For Thermal Transmittance And Air Infiltration Of Garage Doors.

ANSI/DASMA 108 – Determination of Structural Performance Under Uniform Static Air Pressure Difference

ANSI/DASMA 115 – Determination of Structural Performance Under Missile Impact and Cyclic Wind Pressure

ANSI/DASMA 163 - R-Value and U-Factor As Applied To A Residential or Commercial Garage Door.

ASTM International (ASTM):

ASTM A653/A653M - Standard Specification for Specification for Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy-Coated (Galvannealed) by the Hot-Dip Process.

ASTM B209 - Standard Specification for Aluminum-Alloy Sheet and Plate.

ASTM B221 - Standard Specification for Aluminum-Alloy Extruded Bars, Rods, Wires, Shapes and Tubes.

ASTM C518 - Standard Test Method for Steady-State Thermal Transmission Properties by Means of the Heat Flow Meter Apparatus.

ASTM E283 - Standard Test Method for Determining Rate of Air Leakage Through Exterior Windows, Curtain Walls, and Doors Under Specified Pressure Differences Across the Specimen

ASTM E330 – Standard Test Method for Structural Performance of Exterior Windows, Doors, Skylights and Curtain Walls by Uniform Static Air Pressure Difference.

ASTM E1886 – Standard Test Method for Performance of Exterior Windows, Curtain Walls, Doors and Impact Protective Systems Impacted by Missile(s) and Exposed to Cyclic Pressure Differentials.

ASTM E1996 – Standard Specification for Performance of Exterior Windows, Curtain Walls, Doors and Impact Protective Systems Impacted by Windborne Debris in Hurricanes.

American Architects Manufacturers Association (AMAA):

AAMA 2604 – Voluntary Specification, Performance Requirements and Test Procedures for Pigmented Organic Coatings on Aluminum Extrusions and Panels (with Coil Coating Appendix).

SYSTEM DESCRIPTION

** NOTE TO SPECIFIER ** Include the following paragraph for exterior doors.

Design doors to withstand:

** NOTE TO SPECIFIER ** Include the following paragraph for exterior doors.

Positive and negative design wind loads in accordance with Building Code.

Positive and negative design wind loads of __ PSF.

** NOTE TO SPECIFIER **  Delete Windborne-Debris Impact Resistance if not required. Refer to Building Code.

Windborne-Debris Impact Resistance:  Provide impact -protective overhead coiling doors that pass ASTM E1886 missile -impact and cyclic -pressure tests according to ASTM E1996 for Wind Zone ___ for basic protection.

** NOTE TO SPECIFIER ** Delete cycle life options not required. 10,000 cycles is standard.

Cycle life of 10,000 cycles.

Cycle life of 25,000 cycles.

Cycle life of 50,000 cycles.

Cycle life of 100,000 cycles.

Cycle life of __ cycles.

Door Opening and Closing:

** NOTE TO SPECIFIER **  Delete operation options not required.

Operation:  Electric.

Operation:  Manual.

Operation:  Chain hoist.

Track and Operating Hardware:

** NOTE TO SPECIFIER **  Delete the track type options not required.

Standard lift type.

Vertical lift type.

High lift type.

Follow Roof/Incline pitch type.

Low headroom type

SUBMITTALS

Submit under provisions of Section 01 30 00 - Administrative Requirements.

Shop Drawings:  Indicate opening dimensions and required tolerances, jamb connection details, anchorage spacing, hardware locations, installation details, and special conditions.

Product Data:  Provide information on components, applications, hardware and accessories.

** NOTE TO SPECIFIER ** Retain one or both paragraphs below. Retaining both paragraphs indicates "Two Stage Samples" process. Delete "Samples for Initial Selection" if colors have already been determined. Delete items not required.

Samples for Initial Selection:  Provide manufacturer's finish charts showing full range of colors and textures available for units with factory applied finishes.

Samples for Verification:  Provide for each type of exposed finish on the following components in manufacturer's standard sizes:

Color Chip Sample.

Glass Chip Sample.

** NOTE TO SPECIFIER ** Include the following for projects requiring LEED certification. Credits are available for the use of recycled materials and also for regional materials if the project is located within a 500-mile radius of the C.H.I. manufacturing facility in Arthur, IL.

Sustainable Design Submittals:

Recycled products:  Indicate percentage of recycled material used in the manufacturing of products and percentage classified as post-consumer.

Regional products:  Indicate location of product manufacturer and distance from manufacturing facility to project site.

Closeout Submittals:  Operation and maintenance data.

QUALITY ASSURANCE

Manufacturer Qualifications:  Manufacturer shall provide an overhead door system capable of withstanding positive and negative wind loads as required by local building code for 10,000 cycles.

Installer Qualifications:  Installer shall be authorized and qualified to install overhead door systems on the type and scope of project specified.

Source Limitations:  Provide overhead sectional doors from one manufacturer for each type of door. Provide operators and other accessories from source acceptable to overhead door manufacturer.

DELIVERY, STORAGE, AND HANDLING

Store products in manufacturer's unopened packaging until ready for installation.

Store and dispose of all materials in accordance with federal, state and local laws.

PROJECT CONDITIONS

Maintain environmental conditions (temperature, humidity and ventilation) within limits recommended by manufacturer for optimum results. Do not install products under environmental conditions outside manufacturer's recommended limits.

WARRANTY

Provide an original of the manufacturer's limited warranty against manufacturing defect and product workmanship. See https://www.chiohd.com/support/warranty for full warranty details.

1-Sided Steel, Non-Insulated & Insulated Doors

Steel Door Sections: Used in residential applications, under normal conditions, against splitting, cracking, or rusting through for as long as the original purchaser owns the home.

Warranty Period: Limited Lifetime

Hardware: For defects in material or workmanship.

Warranty Period: 6 years from date of manufacture.

Springs: For defects in materials or workmanship.

Warranty Period: 3 years from date of manufacture.

- PRODUCTS

MANUFACTURERS

Acceptable Manufacturer:  C.H.I. Overhead Doors, which is located at:  1485 Sunrise Dr.; Arthur, IL 61911; Toll Free Tel:  800-677-2650; Fax:  800-738-5006; Email:  request info (aia@chiohd.com); Web:  http://www.chiohd.com

** NOTE TO SPECIFIER ** Delete one of the following two paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

Substitutions:  Not permitted.

Requests for substitutions will be considered in accordance with provisions of Section 01 60 00 - Product Requirements.

** NOTE TO SPECIFIER **  Delete article if not required. Then delete finish options and window lites not required.

RESIDENTIAL INSULATED STEEL PAN STYLE DOORS

Long Raised Panel Pan Door Sections

Model 4241 by C.H.I. Overhead Doors

Appearance:  Recessed edge and slightly raised interior surface, woodgrain embossed.

Exterior:  24-gauge, 0.022 inch (0.56 mm) polyester painted galvanized steel.

Interior:  Vinyl, laminated to insulation. Plastic retainer strips.

Door Size (WxH):  Standard up to 20 ft 2 inches (6.15 m) x 16 ft 0 inches (4.88 m).

Calculated R-Value:  7.94.

Finish:  White.

Finish:  Almond.

Finish:  Sandstone.

Finish:  Brown.

Finish:  Evergreen.

Finish:  Powder Coat RAL No. _____

Window Lites:  Long Panel Non-Insulated. See Section 2.3.A & B.

** NOTE TO SPECIFIER **  The additional product features apply to Models 2251, 2241, 4251, 4241, 5251, 5951, 2551, 2151, 4151, 2141, and 4141.

Additional Product Features:

Thickness:  2 inches (51 mm).

Insulation: 1-1/2 inches (38 mm) thick, CFC-free polystyrene.

Joints:  Tongue and Groove.

Plank Sections:  18 inches (457 mm) and 21 inches (533 mm) based on overall door height.

End Caps:  20-gauge, 0.034 inch (0.86 mm) galvanized steel, wrap-around cap style. Clinch locked.

Stiles:  20-gauge, 0.034 inch (0.86 mm) galvanized steel. Clinch locked.

Hinges:  16-gauge, 0.055 inch (1.40 mm) galvanized steel standard up to 7 ft 9 inches (2.36 m). 14-gauge, 0.070 inch (1.77 mm) galvanized steel standard at 8 ft 0 inches (2.44 m)

** NOTE TO SPECIFIER **  Delete locking options not required.

Locking:  No lock.

Locking:  One Inside slide lock.

Locking:  Two Inside slide locks.

Locking:  Outside center lock.

Locking:  Double lock bar.

** NOTE TO SPECIFIER **  Delete weather seal option not required.

Weather Seal:  U-shaped bottom astragal with aluminum retainer (standard).

Weather Seal:  Top Seal

Weather Seal:  Header and Jambs. Flexible one-piece vinyl extrusion.

** NOTE TO SPECIFIER **  Delete article if not required. Then delete window lites not required.

WINDOW LITE OPTIONS

Specify window lite pattern. All products are specified from the inside looking out:

Window Lite Pattern: Full width of section. Section(s) ___ from ground.

Window Lite Pattern:  Left side looking out, __ wide by __ high. Starting section ___ from ground.

Window Lite Pattern:  Right side looking out, __ wide by __ high. Starting section ___ from ground.

Window Lite Pattern:  Per attached drawing.

Long Panel Non-Insulated Window Lites

41 inch (1041 mm) x 13 inch (330 mm) window.

Glazing:  1/8 inch (3 mm) plain DSB.

Glazing:  1/8 inch (3 mm) plain tempered.

Glazing:  1/8 inch (3 mm) obscure.

Glazing:  1/8 inch (3 mm) tinted.

Glazing:  1/8 inch (3 mm) glue chip.

Glazing:  1/8 inch (3 mm) polycarbonate.

Glazing:  1/8 inch (3 mm) frosted.

Window Insert: No Insert.

Window Insert: Stockton.

Window Insert: Cascade.

Window Insert: Prairie.

Window Insert: Waterton.

Window Insert: Sherwood.

Window Insert: Sunburst. Must have 2, 3, or 4 windows.

Window Insert: Arched Stockton. Must have 2, 3, 4 or 5 windows.

Window Insert: Madison.

Window Insert: Arched Madison. Must have 2, 3, 4 or 5 windows.

Panel Option: 3 inches (76 mm) Exhaust Port

Panel Option: 4 inches (102 mm) Exhaust Port

Panel Option: Flood Vent

COMPONENTS

** NOTE TO SPECIFIER **  Track Section below applies to all sections except Residential Builder Doors.

Track

** NOTE TO SPECIFIER **  Delete track type options not required.

Track Type:  Standard Lift, 12 inches (305 mm) radius standard.

Track Type:  Vertical Lift.

Track Type:  High Lift.

Track Type:  Follow Roof Line / Incline. Available in pitch increments of 0.5 from 0.5/12 to 12/12. Consult factory if less than 15 inches (381 mm) of headroom, openings more than 300 square feet (27.9 square meters) or pitch greater than 6/12.

Track Type:  Low Head Room.

** NOTE TO SPECIFIER **  Delete mount options not required.

Front Mount:  12 inches (305 mm) Radius, 2 inches (51 mm) Track Only.

Front Mount:  15 inches (381 mm) Radius.

Rear Mount:  12 inches (305 mm) Radius, 2 inches (51 mm) Track Only.

Rear Mount:  15 inches (381 mm) Radius.

** NOTE TO SPECIFIER **  Delete track appearance options not required. Galvanized is standard.

Track Appearance – Galvanized.

Track Appearance – White Powder Coat.

Track Appearance – RAL Powder Coat No._______.

Track Size:  Track mounting and size is based on door size and weight. Lower track is adjustable for weathertight fit. Optional to upgrade to clip angle or continuous when not standard. Track will be minimum 16-gauge, 0.055 inch (1.39mm) galvanized steel. Gauge will increase based on design requirements.  2-inch (51 mm) track for 2 inch (51 mm) rollers or 3 inch (76 mm) track with 3 inch (76 mm) rollers.

Roller Assemblies:  Galvanized steel adjustable roller holders with floating hardened steel bearing rollers, located at top and bottom of each side of each section. Size and type to be determined by the manufacturer based on door size and weight.

Spring Counterbalance:  Helically-wound, oil-tempered torsion springs mounted on cross-header shaft supported by galvanized steel ball bearing end plates and center carrier brackets as required. Springs to be individually calibrated to each door. Spring shafts are hollow or solid based on door size and weight. Counterbalance transferred to doors via aircraft quality braided steel lift cables.

Spring Cycle Life:  10,000 cycles standard. Consult factory for extended life cycles up to 100,000 cycles. There are limitations based on door size and weight.

Electric Operator:

Externally mounted on drive side of door.

** NOTE TO SPECIFIER ** Delete power supply options not required.

Power Supply:  115 Volts AC single phase.

Power Supply:  208/230 Volts AC single or three phase.

Power Supply:  460 Volts AC three phase.

Manually operable in case of power failure.

** NOTE TO SPECIFIER ** Delete control station power option and control station options not required.

Control Station Power:  24 VDC.

Control Station Power:  115 VAC.

Control Station:  Keyed Switch.

Control Station:  Two button (Open / Close) station.

Control Station:  Three button (Open / Stop / Close) station.

** NOTE TO SPECIFIER ** Delete paragraph if no safety device is specified for electrically controlled units or delete safety device options not required.

Safety Reversing Device:

Safety Device:  Photoelectric sensor; detect obstruction and reverse door without requiring door to contact obstruction.

Safety Device:  Electric pneumatic edge; detect obstruction and reverse door upon contact with pneumatic hose.

Safety Device:  Electric edge; detect obstruction and reverse door upon contact with electric strips in vinyl housing.

Safety Device:  Electric edge; fail-safe, self-monitoring.

- EXECUTION

  - EXAMINATION

    - Do not begin installation until work areas have been properly prepared.

    - If preparation is the responsibility of another installer, notify Architect of unsatisfactory conditions before proceeding.

  - INSTALLATION

    - Install door assembly in accordance with manufacturer's instructions.

    - Anchor to adjacent construction without distortion or stress.

    - Securely brace door tracks suspended from structure. Secure tracks to structural members only.

Fit and align door assembly including hardware, level and plumb, to provide smooth operation.

** NOTE TO SPECIFIER ** Include the following paragraph if head and jamb weatherstripping is specified.

Position head and jamb weatherstripping to contact door sections when closed; secure in position.

** NOTE TO SPECIFIER ** Include the following paragraph if electrically operated doors are specified.

Make wiring connections between power supply and operator and between operator and controls.

  - ADJUSTING

    - Adjust to operate smoothly throughout full operating range.

  - PROTECTION

    - Protect installed products until completion of project.

    - Touch-up, repair or replace damaged products before substantial completion.

END OF SECTION